SOP

- Please take 1 set of each component – 1/8’ push-in connectors, 3/8’ elbow connectors, rods, end plates, insulators, current collectors, cooling fin, spacer, bipolar plate and MEA

- Open the packaging and check on the quality and of the component. Make sure everything is in good condition – no deformation, break, scratch, holes or defects.

- Make sure that the quantity of the components as below:

- push-in connectors: 2

- elbow connectors: 2

- rods: 10

- end plates: 2 (top and bottom)

- insulators: 2

- current collectors: 2

- cooling fin: 23

- spacer: 46

- bipolar plate: 44

- MEA: 22

- Align the current collector to the end plate and label the edge of the current collector using marker pen for all 4 corners. Label both top and bottom plate.

- Take a Teflon tape and wrap around 1/8’ push-in connectors and 3/8’ elbow connectors. Wrap 2 full round of tape on to the thread section.

- Tighten the 1/8’ push-in connectors and 3/8’ elbow connectors to the top and bottom plate (as in the picture below).

- Take 10 rods and screw into the bottom plate until the rod indicator (as picture)

- Pre-stacking jig